<q>What is the primary function of an electromechanical relay?

<a>To convert AC voltage to DC voltage

<a>To amplify electrical signals

<+a>To act as an electrically controlled switch

<a>To regulate voltage in a circuit

<q>What component stores energy to maintain the coil’s energized state after disconnecting the external power source?

<a>Diode

<a>Transistor

<+a>Capacitor

<a>Resistor

<q>What is the purpose of the potentiometer dial on the electronic on-delay relay pictured in the passage?

<a>To adjust the voltage rating of the relay coil

<a>To select between AC and DC input voltage

<a>To regulate the current flow through the timing circuit

<+a>To control the on-delay time of the relay

<q>The passage compares latching relays to what other type of switch?

<a>A rotary switch

<a>A switch

<+a>A toggle switch

<a>A limit switch

<q>What is the main advantage of a solid-state relay (SSR) compared to an electromechanical relay?

<a>Ability to handle both AC and DC currents

<a>Higher voltage and current ratings

<+a>Virtually unlimited cycle life and faster response time

<a>Lower cost and simpler design

<q>Why do SSRs generate more heat than similar electromechanical relays when carrying the same current?

<a>The lack of moving parts increases friction

<a>SSRs use less efficient switching mechanisms

<+a>The semiconducting materials have higher resistance

<a>SSRs require a heat sink for proper operation

<q>In the provided schematic diagram, the dashed line represents the influence of the:

<a>Relay housing on the external environment.

<a>High-current power circuit on the relay housing

<+a>Electromagnet coil on the switch mechanism.

<a>Control circuit voltage on the switch contacts.

<q>What do the terms “Form-A” and “Form-B” correspond to in relay terminology?

<a>Normally open and normally closed contacts

<a>High-current and low-current contacts

<+a>Double-pole and single-pole configurations

<a>The number of throws a relay can handle

<q>Which type of relay contact is described as having one moving contact and two stationary contacts it can touch?

<a>Double-pole, single-throw (DPST)

<+a>Single-pole, double-throw (SPDT)

<a>Double-pole, double-throw (DPDT)

<a>Single-pole, single-throw (SPST)

<q>What does the “COM” terminal designation typically represent on an SPDT relay?

<+a>The moving contact of the switch mechanism

<a>The common connection to the electromagnet coil

<a>A designation specific to a particular relay brand

<a>A high-voltage connection point

<q>How does a DPDT relay differ from an SPDT relay in terms of its contact functionality?

<a>A DPDT relay has three separate moving contacts.

<a>There is no functional difference between the two types.

<+a>A DPDT relay offers isolated control of two independent circuits.

<a>A DPDT relay can handle a higher current than an SPDT relay.

<q> What is the purpose of the commutating diode connected in parallel with a relay coil?

<+a>To provide a safe path for current dissipation during de-energization

<a>To block reverse current flow in the control circuit

<a>To increase the magnetic field strength of the coil

<a>To regulate the voltage supplied to the coil

<q>What is the primary factor determining a relay coil rating?

<a>The voltage drops across the coil when energized

<a>The voltage required to overcome the relay’s internal spring tension

<+a>The current needed to activate the iron armature reliably

<a>The maximum current the coil can handle without overheating

<q>Which term refers to a delay that happens after the coil is de-energized?

<a>On-delay

<a>Slow-delay

<a>Fast-delay

<+a>Off-delay

<q>What is the primary function of a transformer?

<a>Amplify AC signals

<a>Convert DC voltage to AC voltage

<a>Rectify AC voltage to DC

<+a>Step up or step down AC voltage

<q>Which law governs the operation of a transformer?

<a>Kirchhoff's Laws of Electromagnetic Induction

<a>Lenz's Law of Electromagnetic Induction

<a>Ohm's Law of Electromagnetic Induction

<+a>Faraday's Law of Electromagnetic Induction

<q>A transformer consists of:

<a>A single coil with a core

<a>Two coils connected in parallel

<+a>Two coils wrapped around a common core

<a>Two coils connected in series

<q>The core of a transformer is usually made of:

<a>Aluminum

<a>Copper

<+a>Iron or steel

<a>Plastic

<q>A transformer with more turns in the secondary winding compared to the primary winding is called a:

<+a>Step-up transformer

<a>Step-down transformer

<a>Autotransformer

<a>Isolation transformer

<q>Transformers are primarily used in:

<+a>AC circuits

<a>DC circuits

<a>Both AC and DC circuits

<a>Neither AC nor DC circuits

<q>Autotransformers are used to:

<a>Step up or step-down voltage without galvanic isolation

<+a>Provide galvanic isolation between primary and secondary circuits

<a>Amplify signals

<a>Convert AC to DC

<q>The efficiency of a transformer is typically:

<a>Very low

<+a>Very high

<a>Depends on the type of transformer

<a>Moderate

<q>The power input to a transformer is:

<a>Can be more or less than the power output

<a>Always greater than the power output

<+a>Always equal to the power output

<a>Always less than the power output

<q>The primary and secondary windings of a transformer are typically made of:

<a>Aluminum

<a>Iron

<a>Vanadium

<+a>Copper

<q>Transformers are used in power distribution systems to:

<a>Both incorrect

<a>Step up voltage for long-distance transmission

<+a>Both correct

<a>Step down voltage for local distribution

<q>Transformers are used in electronic circuits for:

<+a>All answers correct

<a>Position sensing

<a>Signal coupling

<a>Impedance matching

<q>LVDTs and RVDTs are used for:

<+a>Position sensing

<a>Signal coupling

<a>Common-mode voltage isolation

<a>Impedance matching

<q>Transformers can be used for measurement applications to:

<a>None of the above

<a>Step up high voltages for measurement

<+a>Both step-up and step-down high voltages for measurement

<a>Step down high voltages for measurement

<q>What is the primary advantage of polyphase electric circuits over single-phase circuits?

<+a>More continuous power delivery

<a>Higher voltage

<a>Smaller size

<a>Lower current

<q>What is the most common type of polyphase circuit used in industry?

<a>Four-phase

<+a>Three-phase

<a>Six-phase

<a>Two-phase

<q>How are the voltages in a three-phase system phase-shifted from each other?

<+a>120 degrees

<a>270 degrees

<a>180 degrees

<a>90 degrees

<q>What is the term used to describe the order in which the voltages in a polyphase system reach their peak values?

<a>Phase shift

<a>Phase angle

<a>Phase rotation

<+a>Phase sequence

<q>What are the two primary ways to connect the stator windings of a three-phase generator?

<a>Alfa and Delta

<a>Open and Closed

<a>Series and Parallel

<+a>Star and Delta

<q>How do line voltage and phase voltage relate to each other in a Star-connected system?

<a>Line voltage is equal to phase voltage

<+a>Line voltage is √3 times phase voltage

<a>Line voltage is twice phase voltage

<a>Line voltage is 1/√3 times phase voltage

<q>How do line current and phase current relate to each other in a Delta-connected system?

<a>Line current is 1/√3 times phase current

<a>Line current is equal to phase current

<a>Line current is twice phase current

<+a>Line current is √3 times phase current

<q>What is the relationship between the total power in a three-phase system and the power in each phase?

<a>The total power is the cube of the power in each phase

<a>The total power is the square root of three times the power in each phase

<+a>The total power is the sum of the powers in each phase

<a>The total power is three times the power in each phase

<q>What are the three main components of a three-phase induction motor?

<a>Rotor, field, and armature

<a>Stator, armature, and commutator

<+a>Stator, rotor, and enclosure

<a>Field, rotor, and stator

<q>What is the purpose of the stator in an induction motor?

<a>To provide mechanical power

<a>To protect the motor from damage

<+a>To produce the magnetic field

<a>To convert electrical energy into mechanical energy

<q>What is the purpose of the rotor in an induction motor?

<a>To protect the motor from damage

<a>To convert electrical energy into mechanical energy

<a>To produce the magnetic field

<+a>To provide mechanical power

<q> What is the purpose of the enclosure in an induction motor?

<a>To convert electrical energy into mechanical energy

<a>To provide mechanical power

<+a>To protect the motor from damage

<a>To produce the magnetic field

<q>What is the name of the thin metal sheets used to construct the stator core?

<a>Armature

<+a>Laminations

<a>Field

<a>Windings

<q>What is the synchronous speed of a 4-pole induction motor operating at a frequency of 60 Hz?

<+a>1800 RPM

<a>2400 RPM

<a>3600 RPM

<a>1200 RPM

<q>What is the term for the difference in speed between the stator's rotating magnetic field and the rotor's speed?

<a>Efficiency

<a>Power factor

<a>Torque

<+a>Slip

<q>What is the most common type of induction motor?

<a>Wound-rotor

<a>Synchronous

<+a>Squirrel-cage

<a>DC

<q>How can the direction of rotation of an induction motor be reversed?

<a>By changing the frequency of the power supply

<a>By reversing the direction of the rotor

<a>By reversing the polarity of the power supply

<+a>By swapping two of the power lines

<q>What is the relationship between the number of poles in an induction motor and its synchronous speed?

<a>The number of poles does not affect speed

<+a>More poles mean lower speed

<a>The relationship depends on the motor's type

<a>More poles mean higher speed

<q>What is the term for the maximum continuous torque that an induction motor can produce?

<+a>Full load torque

<a>Starting torque

<a>Breakdown torque

<a>Maximum torque

<q>When two or more bare conductors touch causing the resistance between the two to drop

<a>Ground fault

<+a>Short circuit

<a>Electrical shock

<a>Overload

<q>When the current exceeds the trip class of the device?

<a>Ground fault

<+a>Overload

<a>Short circuit

<a>Electrical shock

<q>What is the primary purpose of a 4-20 mA signal in industrial automation?

<+a>To represent a physical measurement or control signal

<a>To provide a reference voltage for analog circuits

<a>To transmit digital data between devices

<a>To power field instruments

<q>What are the two primary signal standards used in industrial automation?

<a>CANbus and Modbus

<a>1-5V DC and 10-50V DC

<+a>3-15 PSI and 4-20 mA DC

<a>RS-232 and RS-485

<q>What is the primary advantage of a "live zero" signal standard?

<a>It reduces the risk of signal interference

<+a>It provides a simple way to detect signal failures

<a>It increases the signal's dynamic range

<a>It allows for higher signal frequencies

<q>What is the purpose of an I/P transducer?

<a>To filter out noise from the 4-20 mA signal

<a>To amplify a weak electrical signal

<+a>To convert a 4-20 mA current signal to a pneumatic pressure signal

<a>To convert a digital signal to an analog signal

<q>What is the primary difference between a 4-wire transmitter and a 2-wire transmitter?

<a>The 4-wire transmitter is more susceptible to noise interference

<+a>The 4-wire transmitter requires a separate power supply

<a>The 4-wire transmitter has a higher accuracy

<a>The 4-wire transmitter is more expensive

<q>What is the purpose of a precision resistor in a 4-20 mA system?

<+a>To convert a current signal to a voltage signal

<a>To filter out high-frequency noise

<a>To protect the controller from overvoltage

<a>To limit the current flow

<q>What is the typical voltage range for a 4-20 mA signal?

<a>1-5V DC

<a>0-5V DC

<a>0-10V DC

<+a>10-50V DC

<q>What is the relationship between the process variable (PV) and the manipulated variable (MV) in a control loop?

<a>The PV is always greater than the MV

<a>The PV is always less than the MV

<a>The PV and MV are always equal

<+a>The relationship between the PV and MV depends on the controller's algorithm

<q>What is the role of a controller in a control loop?

<+a>To compare the SP to PV and adjust the output accordingly

<a>To actuate the final control element

<a>To power the field instruments

<a>To measure the process variable

<q>What is the purpose of a reverse-acting controller?

<a>To reverse the direction of the FCE

<a>To increase the gain of the controller

<a>To invert the polarity of the 4-20 mA signal

<+a>To decrease the gain of the controller

<q>What is the difference between a direct-acting controller and a reverse-acting controller?

<a>The direct-acting controller is more accurate than the reverse-acting controller.

<a>The direct-acting controller decreases the output when the PV increases, while the reverse-acting controller increases the output when the PV increases.

<a>The direct-acting controller is less expensive than the reverse-acting controller.

<+a>The direct-acting controller increases the output when the PV increases, while the reverse-acting controller decreases the output when the PV increases.

<q>What is the fundamental principle of instrumentation system troubleshooting?

<+a>An instrument's output should correspond to its input.

<a>All instruments require a constant power supply.

<a>All instruments have equal input and output impedance.

<a>All instruments have a single input and a single output.

<q>What is the primary tool for troubleshooting 4-20 mA analog signal-based instruments?

<a>Spectrum analyzer

<a>Oscilloscope

<+a>Multimeter

<a>Logic analyzer

<q>What is the major disadvantage of using a milliammeter to measure 4-20 mA signals?

<a>It requires specialized training to use.

<a>It can introduce noise into the signal.

<+a>It interrupts the signal flow.

<a>It can damage the instrument being measured.

<q>What is the purpose of a clamp-on milliammeter?

<+a>To measure current without breaking the circuit.

<a>To measure resistance without breaking the circuit.

<a>To measure voltage without breaking the circuit.

<a>To measure high-frequency signals.

<q>What is a test diode used for in a 4-20 mA loop?

<+a>To allow for non-invasive current measurement.

<a>To protect the transmitter from overvoltage.

<a>To filter out noise in the signal.

<a>To increase the signal strength.

<q>Why is it difficult to accurately measure current in a 4-20 mA loop using only a voltmeter?

<a>Voltmeters can introduce noise into the circuit.

<a>Voltmeters can only measure DC voltage.

<+a>The voltage drop across the load is often unknown.

<a>Voltmeters are not accurate enough for low-current measurements.

<q>What is a common problem with loop-powered 4-20 mA transmitters?

<+a>They may not have enough power to operate.

<a>They are sensitive to temperature changes.

<a>They are susceptible to noise interference.

<a>They require frequent calibration.

<q>What is the purpose of a loop calibrator?

<a>To calibrate the transmitter.

<+a>To test and troubleshoot 4-20 mA loops.

<a>To calibrate the controller.

<a>To power the loop-powered instruments.

<q>What is the most important factor to consider when troubleshooting a 4-20 mA loop?

<a>The resistance of the wiring.

<+a>The current flowing through the loop.

<a>The voltage drops across the load.

<a>The power supply voltage.

<q>How does the frequency in the Nyquist plot get varied

<a>-∞ to 0

<a>0 to +∞

<+a> -∞ to + ∞

<a>Zero

<q>On what principle is Nyquist stability based?

<a>Principle of Refraction

<a>Principle of Reflection

<+a> Principle of Argument

<a>Principle of Total Internal Reflection

<q>_______ used in determining the stability of the control system?

<a>Bode Plot

<+a> Nyquist Plot

<a>Polar Plot

<a>Polar Plot

<q>If the system matrix of a linear time invariant continuous system is given by its characteristic equation is given by:

<+a> s^2 + 5s + 3

<a>s^2 - 3s - 5

<a>s^2 + 3s + 5

<a>s^2 + s + 2

<q>If the Nyquist plot of the loop transfer function G(s)H(s) of a closed-loop system encloses the (1, j0) point in the G(s)H(s) plane, the gain margin of the system is:

<a>Zero

<a>Greater than zero

<+a> Less than zero

<a>Infinity

<q>What if the enclosed path consists of poles but not zeroes then the direction encircled is?

<+a> Opposite in Direction of Enclosed Closed Path

<a>Encircling is Impossible

<a>Same in Direction of Enclosed Closed Path

<a>Both Poles and Zeroes are Required

<q>Poles present on left-half of plane S of a Closed-loop Transfer function indicates ________?

<a>Closed Loop Control System is Unstable

<a>Closed Loop Control System is Partially Stable

<a>None

<+a> Closed Loop Control System is Stable

<q>Nyquist plots have magnitude as one at a certain frequency referred to as ____________?

<+a> Gain Cross-Over Frequency

<a>Phase Margin

<a>Phase Cross-Over Frequency

<a>Gain Margin

<q>Control system gets stable if?

<+a> Phase Cross Over Frequency > Gain Cross Over Frequency

<a>Phase Cross Over Frequency equal Gain Cross Over Frequency

<a>Phase Cross Over Frequency < Gain Cross Over Frequency

<a>Phase Cross Over Frequency ≥ Gain Cross Over Frequency

<q>If Phase Cross Over Frequency <+a> Gain Cross Over Frequency indicates the system?

<a>Stable

<+a> Marginally Stable

<a>Unstable

<a>None

<q>A linear time invariant system is stable if:

<a>System in excited by the bounded input, the output is also bounded

<a>In the absence of input output tends to zero

<+a> Both a and b

<a>System in excited by the bounded input, the output is not bounded

<q>Asymptotic stability is concerned with:

<a>A system under influence of input

<+a>A system not under influence of input

<a>A system under influence of output

<a>A system not under influence of output

<q>If a system is given unbounded input then the system is:

<a>Stable

<a>Unstable

<+a> Not defined

<a>Linear

<q>Linear mathematical model applies to:

<a>Linear systems

<+a> Stable systems

<a>Unstable systems

<a>Non-linear systems

<q>For non-linear systems, stability cannot be determined due to:

<a>Possible existence of multiple equilibrium states

<a>No correspondence between bounded input and bounded output stability and asymptotic stability

<a>Output may be bounded for the particular bounded input but may not be bounded for the bounded inputs

<+a>All of the mentioned

<q>Bounded input and Bounded output stability notion concerns with:

<+a> A system under influence of input

<a>A system not under influence of input

<a>A system under influence of output

<a>A system not under influence of output

<q>If the impulse response is absolutely integrable then the system is:

<+a> Absolutely stable

<a>Unstable

<a>Linear

<a>Stable

<q>___________ is a quantitative measure of how fast the transients die out in the system.

<a>Absolutely stable

<a>Conditionally stable

<a>Unstable

<+a> Relative Stability

<q>Routh Hurwitz criterion gives:

<+a> Number of roots in the right half of the s-plane

<a>Value of the roots

<a>Number of roots in the left half of the s-plane

<a>Number of roots in the top half of the s-plane

<q>Routh Hurwitz criterion cannot be applied when the characteristic equation of the system contains coefficients which are:

<a>Exponential function of s

<a>Sinusoidal function of s

<a>Complex

<+a> Exponential and sinusoidal function of s and complex

<q>Consider the following statement regarding Routh Hurwitz criterion:

<a>It gives absolute stability

<a>It gives gain and phase margin

<a>It gives the number of roots lying in RHS of the s-plane

<+a> It gives gain, phase margin and number of roots lying in RHS of the s-plane

<q>Which of the test signals are best utilized by the stability analysis?

<+a> Impulse

<a>Step

<a>Ramp

<a>Parabolic

<q>In an open loop control system, the control action is:

<a>Dependent on the output

<+a> Independent of the desired output

<a>Dependent on feedback

<a>Independent of the input

<q>Which of the following is not a classification of control systems?

<a>SISO and MIMO Control Systems

<a>Open Loop and Closed Loop Control Systems

<+a> Parallel and Series Control Systems

<a>Continuous time and Discrete-time Control Systems

<q>The traffic lights control system is an example of a:

<a>Closed loop control system

<+a> Open loop control system

<a>Continuous control system

<a>Discrete control system

<q>A closed loop control system is also known as:

<a>Open loop system

<+a> Automatic control system

<a>Manual control system

<a>Linear control system

<q>What is the main advantage of a closed loop control system over an open loop control system?

<a>Cost-effectiveness

<+a> Stability and accuracy

<a>Simplicity

<a>None of the above

<q>In a control system, the feedback element is responsible for:

<a>Generating the input signal

<a>Controlling the output

<+a> Providing the error signal

<a>Generating noise

<q>The block diagram of an open loop control system consists of:

<a>Feedback element only

<+a> Controller and Plant

<a>Error detector only

<a>None of the above

<q>Which of the following control systems adjusts the output automatically to match the desired response?

<a>Open loop control system

<+a> Closed loop control system

<a>Continuous control system

<a>Discrete control system

<q>In a control system, what is the main purpose of feedback?

<a>Increase the speed of response

<+a> Improve the accuracy and stability

<a>Decrease the cost

<a>Simplify the system

<q>Which of the following is true for a continuous time control system?

<a>All signals are discrete in time

<a>One or more signals are discrete in time

<+a> All signals are continuous in time

<a>None of the above

<q>A SISO control system stands for:

<+a> Single Input and Single Output

<a>System Input and System Output

<a>Signal Input and Signal Output

<a>None of the above

<q>MIMO control systems have:

<a>One input and one output

<a>Multiple inputs and one output

<a>One input and multiple outputs

<+a> Multiple inputs and multiple outputs

<q>Which of the following types of feedback is used to improve the performance of a control system?

<a>Positive feedback

<+a> Negative feedback

<a>Both A and B

<a>None of the above

<q>The purpose of a controller in a control system is to:

<a>Measure the output

<+a> Produce the actuating signal

<a>Compare the input and output

<a>Provide feedback

<q>Which of the following is an example of a closed loop control system?

<a>Washing machine without a timer

<a>Traffic lights control system

<+a> Air conditioner with a thermostat

<a>Electric fan

<q>What does a negative feedback control system do?

<a>Increases the error signal

<+a> Decreases the error signal

<a>Has no effect on the error signal

<a>None of the above

<q>In control systems, the term 'stability' refers to:

<+a> The ability to maintain the desired output without oscillations

<a>The speed of response

<a>The cost-effectiveness

<a>The simplicity of the system

<q>The process of using a root locus diagram is to:

<a>Measure the system output

<+a> Observe the path of the closed loop poles

<a>Generate the input signal

<a>Control the system directly

<q>Which of the following is true about root locus?

<a>It starts at open loop zeros and ends at open loop poles

<+a> It starts at open loop poles and ends at open loop zeros

<a>It starts at the origin and ends at infinity

<a>It starts at infinity and ends at the origin

<q>The magnitude condition in a root locus is used to find:

<a>The angle of the open loop transfer function

<+a> The value of K for the points on the root locus branches

<a>The phase margin of the system

<a>The damping ratio of the system

<q>Which condition is used to check whether a point exists on the root locus branch?

<a>Magnitude condition

<+a> Angle condition

<a>Phase condition

<a>Frequency condition

<q>In the s-domain, the root locus is:

<a>Symmetrical about the imaginary axis

<+a> Symmetrical about the real axis

<a>Asymmetrical

<a>None of the above

<q>The transfer function of a control system is defined as:

<a>The ratio of output to input in the time domain

<+a> The ratio of output to input in the frequency domain

<a>The product of output and input in the time domain

<a>The product of output and input in the frequency domain

<q>In block diagram reduction, blocks in series can be:

<a>Added together

<+a> Multiplied together

<a>Subtracted from each other

<a>Divided by each other

<q>In block diagram algebra, a summing point is used to:

<a>Multiply signals

<a>Divide signals

<+a> Add or subtract signals

<a>Invert signals

<q>Which rule is used to shift a take-off point before a block to a position after the block?

<a>Series rule

<a>Parallel rule

<a>Feedback rule

<+a> Take-off point shifting rule

<q>For a system with multiple inputs, the transfer function can be found by:

<+a> Considering one input at a time and making the remaining inputs zero

<a>Considering all inputs simultaneously

<a>Ignoring all but the main input

<a>None of the above

<q>The term 'poles' in a transfer function refers to:

<+a> The values of s that make the denominator zero

<a>The values of s that make the numerator zero

<a>The values of s that make the transfer function infinity

<a>The values of s that make the transfer function zero

<q>Zeros in a transfer function are:

<+a> The values of s that make the numerator zero

<a>The values of s that make the denominator zero

<a>The values of s that make the transfer function infinity

<a>The values of s that make the transfer function undefined

<q>In a block diagram, blocks in parallel are:

<+a> Added together

<a>Multiplied together

<a>Subtracted from each other

<a>Divided by each other

<q>A block diagram can represent:

<a>Only linear systems

<a>Only non-linear systems

<+a> Both linear and non-linear systems

<a>None of the above

<q>When reducing a block diagram, the first step is to:

<a>Move summing points

<a>Move take-off points

<+a> Combine blocks in series or parallel

<a>Simplify feedback loops

<q>The characteristic equation of a system is derived from:

<a>The numerator of the transfer function

<+a> The denominator of the transfer function

<a>The product of the numerator and denominator

<a>None of the above

<q>Which of the following is a necessary condition for a transfer function to be physically realizable?

<+a> The degree of the numerator must be less than or equal to the degree of the denominator

<a>The degree of the numerator must be greater than the degree of the denominator

<a>The degree of the numerator must be less than the degree of the denominator

<a>None of the above

<q>In control systems, the term 'steady-state error' refers to:

<a>The error that occurs during the transient state

<+a> The error that occurs after the transient effects have died out

<a>The error that occurs during the oscillatory state

<a>None of the above

<q>The phase margin of a stable system should be:

<+a> Positive

<a>Negative

<a>Zero

<a>None of the above

<q>If the gain margin of a system is negative, the system is:

<a>Stable

<+a> Unstable

<a>Marginally stable

<a>Critically stable

<q>A system with a gain margin greater than 1 and a phase margin greater than 0° is:

<a>Unstable

<+a> Stable

<a>Marginally stable

<a>Critically stable

<q>The Nyquist plot is a plot of:

<a>Magnitude vs. phase

<+a> Real part vs. imaginary part

<a>Frequency vs. magnitude

<a>Frequency vs. phase

<q>In the root locus method, the breakaway points are:

<a>Points where the root locus enters the real axis

<a>Points where the root locus leaves the real axis

<+a> Points where multiple root locus branches meet

<a>Points where the root locus intersects the imaginary axis

<q>The centroid of the asymptotes in a root locus plot is:

<+a> The average of the poles

<a>The average of the zeros

<a>The average of the poles minus the average of the zeros

<a>The average of the zeros minus the average of the poles

<q>The angle of departure in a root locus plot is calculated from:

<+a> The sum of angles of zeros minus the sum of angles of poles

<a>The sum of angles of poles minus the sum of angles of zeros

<a>The product of angles of zeros

<a>The product of angles of poles

<q>In the Bode plot, the slope of a pole at the origin is:

<a>0 dB/decade

<+a> -20 dB/decade

<a>-40 dB/decade

<a>-60 dB/decade

<q>The slope of a zero at the origin in a Bode plot is:

<a>0 dB/decade

<+a> +20 dB/decade

<a>+40 dB/decade

<a>+60 dB/decade

<q>The phase angle of a pole at the origin in a Bode plot is:

<a>0°

<+a> -90°

<a>-45°

<a>-180°

<q>The phase angle of a zero at the origin in a Bode plot is:

<a>0°

<+a> +90°

<a>+45°

<a>+180°

<q>The frequency at which the magnitude plot crosses 0 dB is called:

<+a> Gain crossover frequency

<a>Phase crossover frequency

<a>Resonant frequency

<a>None of the above

<q>The Nyquist plot helps in determining:

<a>Stability of the system

<a>Performance of the system

<+a> Both stability and performance

<a>None of the above

<q>In the Nyquist plot, encirclements of the point -1+j0 indicate:

<+a> The number of right half plane poles of the open loop transfer function

<a>The number of left half plane poles of the open loop transfer function

<a>The number of right half plane zeros of the open loop transfer function

<a>The number of left half plane zeros of the open loop transfer function

<q>The Bode plot of a stable system should have a phase margin of:

<+a> 0° or more

<a>Less than 0°

<a>Exactly 0°

<a>None of the above

<q>The gain margin in a Bode plot is determined by:

<a>The magnitude at the phase crossover frequency

<a>The phase at the gain crossover frequency

<a>The frequency where the phase is -180°

<+a> Both A and C

<q>For a second-order underdamped system, the overshoot depends on:

<a>Natural frequency

<+a> Damping ratio

<a>Gain margin

<a>Phase margin

<q>The Nyquist stability criterion is applicable to:

<+a> Only linear time-invariant systems

<a>Both linear and non-linear systems

<a>Only discrete-time systems

<a>Only continuous-time systems

<q>In the Nyquist plot, if the system has N right half plane poles, then to be stable, the plot must:

<a>Encircle -1 N times in the clockwise direction

<+a> Encircle -1 N times in the counterclockwise direction

<a>Not encircle -1 at all

<a>Encircle -1 N+1 times in the clockwise direction

<q>State space representation of a control system consists of:

<+a> A set of differential equations

<a>A single differential equation

<a>A set of algebraic equations

<a>A single algebraic equation

<q>The state vector in state space representation contains:

<a>Input variables

<a>Output variables

<+a> State variables

<a>All of the above

<q>In state space analysis, the matrix B represents:

<a>State matrix

<+a> Input matrix

<a>Output matrix

<a>Direct transmission matrix

<q>The matrix C in state-space representation is known as:

<a>State matrix

<a>Input matrix

<+a> Output matrix

<a>Direct transmission matrix

<q>The state-space representation can be used for:

<a>Only linear systems

<+a> Both linear and non-linear systems

<a>Only continuous-time systems

<a>Only discrete-time systems

<q>The controllability of a system indicates:

<+a> The ability to control the state variables

<a>The ability to observe the output

<a>The stability of the system

<a>The performance of the system

<q>A system is said to be observable if:

<+a> The state can be determined from the output

<a>The input can be determined from the output

<a>The output can be determined from the state

<a>None of the above

<q>The rank of the controllability matrix is equal to:

<a>The number of input variables

<a>The number of output variables

<+a> The number of state variables

<a>The number of transfer functions

<q>In state space analysis, the rank of the observability matrix should be:

<+a> Equal to the number of state variables

<a>Equal to the number of output variables

<a>Equal to the number of input variables

<a>Equal to the number of poles

<q>In state space representation, the matrix D represents:

<a>State matrix

<a>Input matrix

<a>Output matrix

<+a> Direct transmission matrix

<q>The eigenvalues of the state matrix A in the state-space representation indicate:

<+a> The poles of the system

<a>The zeros of the system

<a>The steady-state error

<a>The gain margin

<q>A state feedback controller is used to:

<+a> Modify the state equation

<a>Modify the output equation

<a>Modify the input matrix

<a>Modify the state transition matrix

<q>The design of a state feedback controller requires:

<+a> Controllability of the system

<a>Observability of the system

<a>Stability of the system

<a>None of the above

<q>The observer design in state space analysis is used for:

<a>Estimating the output from the state

<+a> Estimating the state from the output

<a>Estimating the input from the output

<a>Estimating the transfer function

<q>A full-order state observer estimates:

<+a> All the state variables

<a>Only the measurable state variables

<a>Only the unmeasurable state variables

<a>Only the input variables

<q>The pole placement method in state-space design involves:

<+a> Placing the poles of the closed-loop system at desired locations

<a>Placing the zeros of the closed-loop system at desired locations

<a>Placing the poles of the open-loop system at desired locations

<a>Placing the zeros of the open-loop system at desired locations

<q>The Kalman filter is used for:

<+a> State estimation in the presence of noise

<a>Control in the presence of noise

<a>Observer design in the absence of noise

<a>Controller design in the absence of noise

<q>In state-space design, robustness refers to:

<+a> The ability of the system to remain stable under parameter variations

<a>The ability of the system to reject disturbances

<a>The ability of the system to track reference inputs

<a>The ability of the system to minimize steady-state error

<q>In the context of state-space representation, an uncontrollable system means:

<+a> The system's states cannot be controlled by the inputs

<a>The system's outputs cannot be observed

<a>The system's inputs cannot be controlled

<a>The system's states cannot be observed

<q>What is feedback in a control system?

<a>A method to change the output without input

<+a> A process to compare the output with the desired input

<a>A method to amplify signals

<a>None of the above

<q>In a PID controller, what does PID stand for?

<+a> Proportional-Integral-Derivative

<a>Proportional-Integral-Difference

<a>Proportional-Indicator-Derivative

<a>None of the above

<q>Which of the following is an advantage of a closed-loop control system over an open-loop control system?

<a>Stability

<a>Accuracy

<+a> Both A and B

<a>None of the above

<q>What is a transfer function in control systems?

<+a> The ratio of output to input in Laplace domain

<a>The difference between output and input in time domain

<a>The sum of output and input in Laplace domain

<a>None of the above

<q>What is the purpose of a compensator in a control system?

<+a> To improve the performance of the system

<a>To decrease the system stability

<a>To increase the error rate

<a>None of the above

<q>Which of the following represents the root locus method?

<+a> A graphical method to determine the roots of the characteristic equation

<a>A numerical method to solve differential equations

<a>A method to design a compensator

<a>None of the above

<q>What is the Routh-Hurwitz criterion used for?

<+a> To determine the stability of a system

<a>To calculate the transfer function

<a>To design a PID controller

<a>None of the above

<q>Which of the following is a time-domain specification?

<a>Rise time

<a>Settling time

<a>Steady-state error

<+a> All of the above

<q>What does Bode plot represent in control systems?

<+a> Frequency response of a system

<a>Time response of a system

<a>Stability of a system

<a>None of the above

<q>What is the Nyquist criterion used for?

<+a> To assess the stability of a control system

<a>To design a compensator

<a>To calculate the transfer function

<a>None of the above

<q>What does the term "gain margin" refer to?

<+a> The amount by which the system gain can be increased before it becomes unstable

<a>The amount by which the system gain can be decreased before it becomes unstable

<a>The amount of feedback in the system

<a>None of the above

<q>What is the function of a low-pass filter in control systems?

<+a> To allow low-frequency signals to pass through and attenuate high-frequency signals

<a>To allow high-frequency signals to pass through and attenuate low-frequency signals

<a>To amplify signals

<a>None of the above

<q>Which of the following is a characteristic of a second-order control system?

<a> It has two poles

<a>It has a quadratic characteristic equation

<+a> Both A and B

<a>None of the above

<q>What is the purpose of a lead compensator?

<+a> To increase the phase margin

<a>To decrease the phase margin

<a>To eliminate steady-state error

<a>None of the above

<q>What is meant by "steady-state error"?

<+a> The difference between the desired and actual output as time approaches infinity

<a>The difference between the initial and final output

<a>The rate at which the system reaches steady state

<a>None of the above

<q>What is the main advantage of state-space analysis?

<+a> It can handle multiple-input multiple-output (MIMO) systems

<a>It can only handle single-input single-output (SISO) systems

<a>It is simpler than transfer function analysis

<a>None of the above

<q>Which of the following is a state variable in a control system?

<+a> A variable that represents the state of the system at a given time

<a>A constant parameter in the system

<a>A variable that only exists in the Laplace domain

<a>None of the above

<q>What is the main purpose of the Laplace transform in control systems?

<+a> To transform differential equations into algebraic equations

<a>To solve algebraic equations

<a>To design filters

<a>None of the above

<q>What is MATLAB?

<+a> A high-level programming language for numerical computation

<a>A low-level programming language

<a>A hardware description language

<a>None of the above

<q>Which of the following is a feature of MATLAB?

<a>Matrix operations

<a>Data visualization

<a>Algorithm development

<+a> All of the above

<q>What is Simulink?

<+a> A graphical programming environment for modeling, simulating, and analyzing multidomain dynamical systems

<a>A text-based programming environment

<a>A hardware design tool

<a>None of the above

<q>Which command is used to start Simulink in MATLAB?

<+a> simulink

<a>startsimulink

<a>launchsimulink

<a>None of the above

<q>How can you create a new model in Simulink?

<+a> By clicking on 'New Model' in the Simulink Library Browser

<a>By typing new_model in the MATLAB command window

<a>By typing create_model in the MATLAB command window

<a>None of the above

<q>Which block is used to represent a transfer function in Simulink?

<+a> Transfer Function block

<a>Gain block

<a>Sum block

<a>None of the above

<q>How can you run a Simulink model?

<+a> By clicking the 'Run' button in the Simulink toolbar

<a>By typing run_model in the MATLAB command window

<a>By typing simulate_model in the MATLAB command window

<a>None of the above

<q>What is the use of the Scope block in Simulink?

<+a> To visualize signals

<a>To add two signals

<a>To multiply two signals

<a>None of the above

<q>Which of the following can be modeled using Simulink?

<a>Electrical systems

<a>Mechanical systems

<a>Biological systems

<+a> All of the above

<q>How do you add a block to a Simulink model?

<+a> By dragging it from the Simulink Library Browser into the model window

<a>By typing add_block in the MATLAB command window

<a>By clicking 'Insert Block' in the Simulink toolbar

<a>None of the above

<q>What does the 'Gain' block in Simulink do?

<+a> Multiplies the input signal by a constant value

<a>Adds a constant value to the input signal

<a>Differentiates the input signal

<a>None of the above

<q>How can you save a Simulink model?

<+a> By clicking 'File' and then 'Save'

<a>By typing save_model in the MATLAB command window

<a>By typing save_simulink in the MATLAB command window

<a>None of the above

<q>Which of the following commands is used to create a plot in MATLAB?

<+a> plot

<a>graph

<a>figure

<a>draw

<q>What does the Routh-Hurwitz criterion determine?

<a>The location of poles

<+a> The stability of the system

<a>The transient response

<a>The frequency response

<q>Which type of system has zero steady-state error for a parabolic input?

<a> Type-0 system

<a> Type-1 system

<+a>  Type-2 system

<a> Type-3 system

<q>The phase margin of a system can be increased by:

<a> Adding a pole

<+a>  Adding a zero

<a> Increasing the gain

<a> Decreasing the gain

<q>The stability of a system can be determined by:

<a> The number of zeros

<a> The number of poles

<+a>  The location of poles in the s-plane

<a> The location of zeros in the s-plane

<q>Which of the following methods is used to improve the steady-state accuracy of a control system?

<a> Increasing the proportional gain

<+a>  Adding an integral action

<a> Adding a derivative action

<a> Reducing the system order

<q>The effect of a Proportional-Integral (PI) controller on the system is:

<a> To increase the steady-state error

<a> To improve the transient response

<+a>  To eliminate the steady-state error

<a> To increase the system order

<q>The Nyquist plot is used to determine:

<a> The transient response of a system

<a> The frequency response of a system

<a> The steady-state error of a system

<+a>  The stability of a system

<q>The use of a Bode plot is to:

<a> Determine the transient response

<a> Plot the time response

<+a>  Analyze the frequency response

<a> Determine the stability using the s-plane

<q>In control system analysis, the term 'bandwidth' refers to:

<+a>  The range of frequencies over which the system can respond effectively

<a> The gain margin of the system

<a> The phase margin of the system

<a> The steady-state error

<q>The addition of a derivative action in a controller helps in:

<a> Reducing the steady-state error

<+a>  Improving the transient response

<a> Increasing the steady-state error

<a> Decreasing the system order

<q>In a unity feedback system, if the open-loop transfer function has more poles than zeros, the system is:

<+a>Stable

<a>Unstable

<a>Marginally stable

<a>Critically stable

<q>The transfer function of a lead compensator is characterized by:

<+a>One zero and one pole with the zero further from the origin

<a>One zero and one pole with the pole further from the origin

<a>Two zeros and one pole

<a>Two poles and one zero

<q>Which controller is most effective in reducing the peak overshoot?

<a>Proportional (P) controller

<a>Integral (I) controller

<a>Derivative (D) controller

<+a>Proportional-Derivative (PD) controller

<q>Which of the following is a disadvantage of a Proportional-Integral-Derivative (PID) controller?

<+a>Difficulty in tuning

<a>High steady-state error

<a>Poor transient response

<a>High sensitivity to noise

<q>What is the primary use of a transfer function in control systems?

<a>To plot the time response

<a>To determine the transient response

<+a>To analyze the input-output relationship in the frequency domain

<a>To calculate the steady-state error

<q>Which of the following represents the bandwidth of a system in a Bode plot?

<a>The frequency range over which the gain is above 0 dB

<+a>The frequency range over which the gain is within -3 dB of its maximum value

<a>The frequency at which the phase angle is -180°

<a>The frequency at which the gain margin is zero

<q>The response of a control system that overshoots and oscillates before settling is indicative of:

<a>Overdamped system

<+a>Underdamped system

<a>Critically damped system

<a>Marginally stable system

<q>In a second-order system, the term 'natural frequency' refers to:

<a>The frequency at which the system oscillates with damping

<a>The frequency of the input signal

<+a>The undamped frequency of oscillation

<a>The frequency at which the system is unstable

<q>Which of the following compensators is used to improve the phase margin of a system?

<a>Lag compensator

<+a>Lead compensator

<a>PI compensator

<a>PD compensator

<q>Which of the following is a type of control system?

<a>Open-loop control system

<a>Closed-loop control system

<+a>Both A and B

<a>None of the above

<q>The error signal in a closed loop control system is:

<a>The sum of the input and feedback signal

<+a>The difference between the input and feedback signal

<a>The product of the input and feedback signal

<a>None of the above

<q>linear time invariant system Stability of a system implies that :

<a>Small changes in the system input does not result in large change in system output

<a>Small changes in the system parameters does not result in large change in system output

<a>Small changes in the initial conditions does not result in large change in system output

<+a>All of the above mentioned

<q>A linear time invariant system is stable if :

<a>System in excited by the bounded input, the output is also bounded

<a>In the absence of input output tends zero

<+a>Both a and b

<a>System in excited by the bounded input, the output is not bounded

<q>Asymptotic stability is concerned with:

<a>A system under influence of input

<+a>A system not under influence of input

<a>A system under influence of output

<a>A system not under influence of output

<q>What is MATLAB?

<+a>A high-level programming language for numerical computation

<a>A low-level programming language

<a>A hardware description language

<a>None of the above

<q>Which of the following is a feature of MATLAB?

<a>Matrix operations

<a>Data visualization

<a>Algorithm development

<+a> All of the above

<q>What is Simulink?

<+a>A graphical programming environment for modeling, simulating, and analyzing multidomain dynamical systems

<a>A text-based programming environment

<a>A hardware design tool

<a>None of the above

<q>What is the use of the Scope block in Simulink?

<+a>To visualize signals

<a>To add two signals

<a>To multiply two signals

<a>None of the above

<q>How do you add a block to a Simulink model?

<+a>By dragging it from the Simulink Library Browser into the model window

<a>By typing add_block in the MATLAB command window

<a>By clicking 'Insert Block' in the Simulink toolbar

<a>None of the above

<q>What does the 'Gain' block in Simulink do?

<+a>Multiplies the input signal by a constant value

<a>Adds a constant value to the input signal

<a>Differentiates the input signal

<a>None of the above

<q>What does the command clear do in MATLAB?

<+a>Clears all variables from the workspace

<a>Clears the command window

<a>Clears the current figure

<a>None of the above

<q>How do you define a matrix in MATLAB?

<+a>Using square brackets, e.g., [1 2; 3 4]

<a>Using curly braces, e.g., {1 2; 3 4}

<a>Using parentheses, e.g., (1 2; 3 4)

<a>None of the above

<q>What is the use of the subplot command in MATLAB?

<+a>To create multiple plots in a single figure

<a>To create a subplot of an existing plot

<a>To add a legend to a plot

<a>None of the above

<q>What is an input signal in an automatic control system?

<a>A signal arriving at the output of the system

<+a>A signal arriving at the input of the system

<a>Signal processed by the controller

<a>Feedback signal

<q>What is a transfer function in automatic control theory?

<a>A function describing the response of a system to an external influence

<+a> A function describing the relationship between the input and output of a system

<a>A function describing the parameters of the regulator

<a>Function describing the dynamics of the control object

<q>What does the MATLAB command 'plot' do?

<a>Prints text to the command window

<a>Saves the workspace

<+a> Creates a 2D plot of data

<a>Clears the workspace

<q>Which command is used to clear the command window in MATLAB?

<a>clear

<+a>clc

<a>clf

<a>close

<q>How are logarithmic frequency characteristics obtained in MATLAB?

<a>nyquist

<+a>bode

<a>step

<a>impulse

<q>What does the feedback function in MATLAB do?

<a>Applies positive feedback to a system

<a>Removes feedback from a system

<+a>Applies negative feedback to a system

<a>Inverts the feedback signal

<q>Which package in MATLAB is used for creating linear models of automatic control systems?

<a>Image Processing Toolbox

<a>Simulink

<+a> Control System Toolbox

<a>Statistics Toolbox

<q>Which software is primarily used in the laboratory practicals for studying ACS?

<a>AutoCAD

<+a>MATLAB

<a>SolidWorks

<a>Python

<q>What does an operational amplifier amplify?

<a>Current

<+a>Voltage

<a>Frequency

<a>Resistance

<q>Which terminal of an op-amp is non-inverting?

<a>Negative

<+a>Positive

<a>Ground

<a>Output

<q>A sawtooth wave is characterized by:

<a>Only sine components

<+a>Linear rise and abrupt fall

<a>Random noise

<a>Flat top pulse

<q>A Bipolar Junction Transistor has how many terminals?

<a>2

<+a>3

<a>4

<a>5

<q>In a regulated power supply rectifier stage converts:

<a>DC to AC

<+a>AC to DC

<a>DC to DC

<a>Frequency to voltage

<q>A 555 timer in astable mode generates:

<a>One pulse

<+a>Continuous pulses

<a>No output

<a>DC voltage

<q>The trigger pin of a 555 timer is:

<a>Pin 1

<+a>Pin 2

<a>Pin 3

<a>Pin 8

<q>In op-amp mathematical operations, summing amplifier performs:

<a>Multiplication

<+a>Addition

<a>Division

<a>Filtering

<q>A transistor in cutoff region acts as:

<+a>Open switch

<a>Closed switch

<a>Amplifier only

<a>Oscillator

<q>In a delay timer circuit, the time delay depends on:

<a>Resistor only

<a>Capacitor only

<+a>RC product

<a>Supply voltage

<q>A regulated power supply reduces:

<+a>Ripple voltage

<a>Load current

<a>Frequency

<a>Resistance

<q>A bistable multivibrator is also called:

<a>Oscillator

<+a>Flip-flop

<a>Rectifier

<a>Amplifier

<q>In op-amp, negative feedback:

<a>Increases distortion

<+a>Reduces gain

<a>Burns the circuit

<a>Disconnects input

<q>The sawtooth generator uses:

<a>Transformer

<+a>Integrator

<a>Rectifier

<a>Inductor

<q>A BJT transistor type NPN means current flows:

<+a>Emitter to base

<a>Collector to emitter

<a>Base to collector

<a>Input to output

<q>A 555 timer powered between 4.5–15 V means it is:

<a>Voltage amplifier

<+a>Timer IC

<a>Comparator

<a>Memory chip

<q>Op-amp ideal input impedance is:

<a>Zero

<+a>Infinite

<a>Very small

<a>Negative

<q>In delay timer, a negative pulse at trigger causes:

<a>Reset

<+a>Trigger action

<a>Output off

<a>Short circuit

<q>A regulated power supply output is:

<a>Unstable AC

<a>Pulsating DC

<+a>Constant DC

<a>Random noise

<q>Bistable multivibrator outputs:

<a>One stable state

<+a>Two stable states

<a>Three states

<a>None

<q>What is differential amplification?

<a>Amplifying sum of voltages

<+a>Amplifying difference between inputs

<a>Frequency amplification

<a>Power regulation

<q>In op-amp subtraction circuit, output is:

<a>Sum

<+a>Difference

<a>Product

<a>Rate of change

<q>Sawtooth wave generator needs:

<a>Comparator only

<+a>Integrator and feedback

<a>Only rectifier

<a>Only transistor biasing

<q>In a 555 astable circuit, the output waveform is:

<a>Sawtooth

<a>Triangle

<+a>Square

<a>Sine

<q>In a BJT amplifier, gain is ratio of:

<+a>Output current to input current

<a>Output voltage to input voltage

<a>Input voltage to supply voltage

<a>Collector to base

<q>Op-amp slew rate limits:

<a>Low frequency only

<+a>High frequency performance

<a>DC gain

<a>Power supply only

<q>In regulated power supply, zener diode is used for:

<a>Rectification

<a>Filtering

<+a>Voltage reference

<a>Oscillation

<q>A delay timer circuit will hold output high until:

<+a>Time expires

<a>Load changes

<a>Power shuts

<a>Frequency changes

<q>Op-amp inverting amplifier gain depends on:

<a>Input only

<+a>Resistor ratio

<a>Capacitor value

<a>Supply voltage

<q>The bistable multivibrator changes state by:

<a>Reset only

<+a>Trigger inputs

<a>Random noise

<a>RC network

<q>Sawtooth waveform amplitude is affected by:

<a>Supply voltage only

<a>Input sine amplitude

<+a>RC time constant

<a>Frequency meter

<q>In a 555 timer astable, duty cycle depends on:

<a>RC only

<a>Internal comparator value

<+a>External resistors and capacitor values

<a>Input signal

<q>Op-amp integrator output is proportional to:

<a>Time derivative

<+a>Time integral of input

<a>Input amplitude only

<a>Frequency

<q>In BJT, saturation means:

<a>Transistor off

<+a>Transistor fully on

<a>Amplifier mode

<a>Reverse bias

<q>Regulated power supply filter reduces ripple by:

<a>Inductance

<+a>Capacitance

<a>RC network

<a>High frequency

<q>The 555 timer comparators compare internal reference voltages of:

<a>1/2 Vcc only

<+a>1/3 and 2/3 Vcc

<a>1/4 Vcc

<a>Ground only

<q>A summing amplifier produces output equal to:

<a>Input average

<+a>Weighted sum

<a>Differential

<a>Times constant

<q>The threshold pin of 555 triggers a change when voltage:

< 1/3 Vcc

<+a>>2/3 Vcc

<a>= supply

<a>= ground

<q>Op-amp comparator output saturates at:

<a>Middle voltage

<+a>Supply rails

<a>Zero

<a>Negative only

<q>Bistable output is set by:

<a>RC network

<a>Comparator only

<+a>S input

<a>Threshold

<q>In sawtooth generator, linearity requires:

<+a>Constant current charge

<a>Variable resistance

<a>Transformer coupling

<a>Inductor feedback

<q>Op-amp differential gain is defined as:

<a>Gain for common signals

<+a>Gain for difference signals

<a>Input bias

<a>Output impedance

<q>In BJT amplifier, why biasing is required?

<a>To control frequency

<+a>To set correct quiescent point

<a>To invert signal

<a>To filter noise

<q>The 555 internal flip-flop switches state when:

<+a>Trigger crosses thresholds

<a>Power shuts

<a>Comparator fails

<a>Output load changes

<q>Regulated power supply dropout voltage refers to:

<a>Noise level

<+a>Minimum supply to maintain regulation

<a>Output ripple

<a>Load current

<q>Input offset voltage in op-amp affects:

<a>Low frequency only

<+a>DC accuracy

<a>High frequency

<a>Output power

<q>In delay timer mode, 555 output remains high until capacitor voltage:

<a>Reaches 1/2 Vcc

<+a>Reaches 2/3 Vcc

<a>Drops to ground

<a>Oscillates

<q>A regulated power supply uses which component to reduce ripple?

<a>Diode

<a>Inductor only

<+a>Capacitor filter

<a>Transformer stripper

<q>Op-amp common-mode rejection is important to:

<a>Amplify all signals

<+a>Reject common voltage noise

<a>Increase output ripple

<a>Reduce gain

<q>In 555 astable mode, frequency formula includes:

<a>Only R1

<+a>R1 and R2 and C

<a>Supply voltage

<a>Trigger level only

<q>Op-amp output depends on the difference between:

<+a>Inputs

<a>Power rails

<a>Frequency

<a>Ground

<q>Sawtooth output has:

<a>Random shape

<+a>Linear ramp

<a>Pure sine

<a>Constant DC

<q>BJT base controls:

<a>Output only

<a>Emitter current

<+a>Collector current

<a>All

<q>A regulated power supply requires:

<a>Rectifier

<a>Filter

<a>Regulator

<+a>All

<q>Reverse proportional parameter of frequency:

<+a>Period

<a>Tough

<a>Torque

<a>Speed

<q>The output pin of a 555 timer is:

<a>Pin 1

<a>Pin 2

<+a>Pin 3

<a>Pin 8

<q>A bistable multivibrator has:

<a>No stable state

<a>One stable state

<+a>Two stable states

<a>Infinite states

<q>In an op-amp, the symbol "+" represents:

<a>Inverting input

<+a>Non-inverting input

<a>Output

<a>Ground

<q>A rectifier circuit mainly uses:

<a>Transistors

<a>Capacitors

<+a>Diodes

<a>Inductors

<q>The emitter terminal of a BJT is usually:

<a>Heavily doped

<+a>Lightly doped

<a>Undoped

<a>Insulated

<q>A sawtooth waveform has:

<a>Exponential rise

<+a>Linear rise and sudden fall

<a>Sinusoidal shape

<a>Constant amplitude

<q>In a regulated power supply, filtering is done using:

<a>Transformer

<+a>Capacitor

<a>Zener diode

<a>Transistor

<q>An op-amp ideally amplifies:

<a>Input current

<a>Output resistance

<+a>Difference between inputs

<a>Supply voltage

<q>The trigger pin of a 555 timer activates when voltage is:

<a>Above Vcc

<+a>Below 1/3 Vcc

<a>At ground

<a>Above 2/3 Vcc

<q>A transistor used as a switch operates in:

<a>Active region

<+a>Cutoff and saturation

<a>Breakdown

<a>Linear mode

<q>A bistable circuit is commonly used as:

<a>Oscillator

<+a>Memory element

<a>Amplifier

<a>Rectifier

<q>The main purpose of a regulator is to:

<a>Increase current

<+a>Stabilize voltage

<a>Reduce frequency

<a>Amplify signal

<q>In astable mode, 555 timer output is:

<a>DC

<+a>Pulsating square wave

<a>Triangle wave

<a>Noise

<q>The collector current in a BJT is controlled by:

<a>Collector voltage

<+a>Base current

<a>Emitter voltage

<a>Temperature

<q>An ideal op-amp has output impedance:

<a>Infinite

<a>Very high

<+a>Zero

<a>Variable

<q>In a summing amplifier, multiple inputs are combined using:

<a>Capacitors

<+a>Resistors

<a>Inductors

<a>Transformers

<q>The frequency of a sawtooth generator mainly depends on:

<a>Power supply

<+a>RC time constant

<a>Load resistance

<a>Output current

<q>In a BJT amplifier, biasing ensures:

<a>No output

<+a>Proper operating point

<a>Maximum noise

<a>Oscillation

<q>The output of an integrator op-amp is proportional to:

<a>Input derivative

<+a>Input integral

<a>Input frequency

<a>Input power

<q>A 555 timer in monostable mode acts as:

<a>Oscillator

<+a>One-shot pulse generator

<a>Flip-flop

<a>Amplifier

<q>The duty cycle of a 555 astable circuit depends on:

<a>Supply voltage

<a>Internal transistor

<+a>External resistors and capacitor

<a>Load current

<q>In a regulated power supply, ripple factor is reduced by:

<a>Rectifier

<+a>Filter

<a>Transformer

<a>Regulator

<q>A comparator compares:

<a>Two currents

<+a>Two voltages

<a>Two resistances

<a>Two frequencies

<q>A non-inverting amplifier provides:

<a>Phase inversion

<+a>No phase inversion

<a>Frequency shift

<a>Power amplification

<q>The bistable multivibrator changes state when:

<a>Power is removed

<+a>Trigger input is applied

<a>Capacitor discharges

<a>Frequency changes

<q>In op-amps, common-mode signals are ideally:

<a>Amplified

<+a>Ignored

<a>Rectified

<a>Integrated

<q>The main function of a zener diode is:

<a>Amplification

<+a>Voltage regulation

<a>Rectification

<a>Switching

<q>In a BJT, saturation region means:

<a>Transistor OFF

<+a>Transistor fully ON

<a>Amplifier mode

<a>Reverse bias

<q>The output of a comparator is usually:

<a>Linear

<+a>Digital-like

<a>Saturated

<a>Integrated

<q>The sawtooth wave generator uses an op-amp as:

<a>Comparator

<+a>Integrator

<a>Rectifier

<a>Buffer

<q>In a delay timer, delay time increases if:

<a>R decreases

<a>C decreases

<+a>R or C increases

<a>Supply decreases

<q>Op-amp offset voltage mainly affects:

<a>AC gain

<+a>DC accuracy

<a>Frequency response

<a>Output power

<q>The reset pin of a 555 timer is used to:

<a>Start oscillation

<+a>Stop output

<a>Change frequency

<a>Increase gain

<q>A regulated power supply without regulator is called:

<a>Switching supply

<+a>Unregulated supply

<a>Linear supply

<a>SMPS

<q>In a differential amplifier, output depends on:

<a>Sum of inputs

<+a>Difference of inputs

<a>Input frequency

<a>Supply voltage

<q>High slew rate in op-amp allows:

<a>Better DC accuracy

<+a>Faster output voltage change

<a>Higher input impedance

<a>Lower noise

<q>The linearity of sawtooth waveform improves with:

<a>Variable current charging

<+a>Constant current charging

<a>High ripple

<a>Inductive load

<q>Common Mode Rejection Ratio (CMRR) indicates ability to:

<a>Amplify noise

<+a>Reject common signals

<a>Increase gain

<a>Reduce bandwidth

<q>In 555 timer, internal reference voltages are set at:

<a>1/2 Vcc

<+a>1/3 and 2/3 Vcc

<a>1/4 and 3/4 Vcc

<a>Vcc only

<q>Dropout voltage of a regulator is:

<a>Output ripple

<+a>Minimum input-output difference

<a>Load current

<a>Switching loss

<q>In bistable multivibrator, output remains unchanged until:

<a>RC discharges

<+a>Trigger is applied

<a>Frequency changes

<a>Power fluctuates

<q>Op-amp integrator becomes unstable at high frequency due to:

<a>Offset voltage

<a>Noise

<a>DC gain

<+a>Saturation

<q>In BJT, thermal runaway occurs due to increase in:

<a>Base resistance

<+a>Collector current

<a>Supply voltage

<a>Gain bandwidth

<q>The regulation percentage indicates:

<a>Output power

<+a>Load variation effect

<a>Efficiency

<a>Ripple voltage

<q>In 555 astable mode, frequency is inversely proportional to:

<a>R only

<a>C only

<+a>RC combination

<a>Supply voltage

<q>An op-amp is mainly used for:

<a>Power generation

<+a>Signal amplification

<a>Rectification

<a>Heating

<q>A BJT has terminals named:

<a>Gate, Source, Drain

<+a>Base, Emitter, Collector

<a>Anode, Cathode

<a>Input, Output

<q>A sawtooth wave has a:

<a>Curved rise

<+a>Linear ramp

<a>Constant value

<a>Random shape

<q>A rectifier converts:

<+a>AC to DC

<a>DC to AC

<a>AC to AC

<a>DC to DC

<q>A 555 timer IC contains:

<a>One comparator

<+a>Two comparators

<a>Three comparators

<a>No comparator

<q>In a bistable multivibrator, memory is stored as:

<+a>Voltage level

<a>Frequency

<a>Phase

<a>Current slope

<q>The main source of ripple is:

<a>Filter

<+a>Rectifier

<a>Regulator

<a>Load

<q>The inverting input of an op-amp is marked with:

<a>+

<+a>-

<a>0

<a>OUT

<q>Astable multivibrator produces:

<a>Single pulse

<+a>Continuous pulses

<a>No output

<a>DC

<q>A zener diode operates in:

<a>Forward region

<+a>Reverse breakdown

<a>Saturation

<a>Cutoff

<q>In a BJT, base current is:

<a>Zero

<+a>Very small

<a>Very large

<a>Infinite

<q>Filtering mainly removes:

<a>DC

<+a>AC ripple

<a>Load current

<a>Voltage drop

<q>The output of a comparator is:

<a>Analog

<+a>Digital-like

<a>Sinusoidal

<a>Integrated

<q>An ideal op-amp has infinite:

<a>Output impedance

<+a>Input impedance

<a>Bandwidth

<a>Offset voltage